- Purpose 目的：

The process of a semi-product inspection is to ensure the drying process is effective and the semi-product after drying shows acceptable appearances and moisture/water activity contents. 半成品检查的过程是为了确保干燥过程有效，并且干燥后的半成品呈现出可接受的外观和水分/水活性含量。

- Scope 范围

Applicable to all products that are dried from the dry room and waiting for the packaging process.  适用于从干燥室干燥并等待包装过程的所有产品

- Responsibility 职责

  - QC is responsible for the inspection of the semi-products after the drying process, making judgments of the qualities of these products, providing feedback on products with quality defects, and tracking bad improvements. QC负责检查干燥过程后的半成品，判断这些产品的质量，提供有质量缺陷的产品的反馈，并跟踪不良改进

  - The production line is responsible for product drying time management, equipment management, dry room cleaning, as well as other aspects related to product drying process. 生产线负责产品干燥时间管理，设备管理，干燥室清洁，以及与产品干燥过程相关的其他方面

- Procedures 流程

  - Check核对

According to the Appendix III "Product Drying Standard", check the actual product's in and out dry room time to make sure the products are dried as scheduled time.    根据“产品干燥标准”，检查实际产品的进出干燥室时间，确保产品按计划时间干燥。

  - Record记录

Fill in form QCD-F07-D [Semi-Product Sample for Testing] to track below information including the following: WT#, Item #, LOT, cart#, Qty, shift, machine, MFG date, and dry out date.  At the same time make a "/" mark on each cart.  填写表格QCD-F07-D [半成品样品测试]，跟踪以下信息，包括以下内容：WT＃，物品＃，批号，车号，数量，班次，机器，制造日期和干燥日期。 同时在每个货车上打“/”号。

  - Take Samples取样

Samples need to be taken separately for a semi-product appearance inspection and water activity/moisture testing. 样品需要分别进行半成品外观检查和水分/水活性测试。

    - Sampling location: An even drying is required to ensure a representative sample is taken. However, the current drying process cannot guarantee an even drying throughout the entire layers, and therefore, sampling needs to be done from the wettest place (mid-lower layers, center of the tray). 取样位置：需要进行均匀干燥以确保取得代表性样品。然而，当前的干燥过程无法保证整个层面的均匀干燥，因此，取样需要从最潮湿的地方进行（中下层，托盘中心）。

    - Sampling plan for appearance inspection  外观检验的抽样计划

The sampling plan for the product appearance inspection follows Military Standard 105E general inspection level II with an acceptable quality level (AQL) 2.5%. For example, TruRX M contains 240 bones in each tray and 32 bones need to be checked for defects. If 3 bones show major defects, this tray should be rejected and the supervisor needs to be notified.  产品外观检验的抽样计划遵循《Military Standard 105E》的II级一般检验水平，接受质量水平（AQL）为2.5%。例如，TruRX M每个托盘含有240根骨头，需检查32根骨头是否有缺陷。如果有3根骨头显示主要缺陷，则应拒绝此托盘，并通知主管。

    - Sampling plan for the water activity and moisture testing 水分活度和水分测试的抽样计划

QC lab testing sampling follows Appendix IV [QC Lab Testing Sampling Fundamental Rules]. QC实验室测试抽样遵循 附录4 [QC实验室测试抽样基本规则]。

- PTR: If carts number is ≤ 4, take 1PCS sample from last cart of those carts.  If each item product carts > 4, take one sample from those carts per 4 carts. Until less than 4 carts, take one sample for last cart.

PTR：如果手推车编号≤4，则从这些手推车的最后一个手推车中取1个样品。 如果每个项目产品手推车>4，则从每4个手推车中取一个样品。直到少于4个手推车，最后一个手推车取一个样品。

- All other products including soft treats: If carts number is ≤ 3, take 1PCS sample from last cart of those carts.  If each item product carts > 3, take one sample from those carts per 3 carts. Until less than 3 carts, take one sample for last cart.

包括软食品在内的所有其他产品：如果手推车编号≤3，则从这些手推车的最后一个手推车中取1个样品。 如果每个项目产品手推车>3，则从每3个手推车中取一个样品。直到少于3个手推车，最后一个手推车取一个样品。

    - Sampling plan for the pH tests of soft treats during production 生产中软食品的pH测试抽样计划

Soft treats process is cold extrusion and therefore, a strict pH testing is critical important for microbiology safety control purposes. 软食品的工艺是冷挤压，因此，严格的pH测试对于微生物安全控制至关重要。

- For all bitable soft treats, 1 mixing sample needs to be taken from every batch during production. A pH > 5.5 results in a rework of the entire batch. 对于所有可咬软食品，在生产过程中每个批次需要取1个混合样品。 如果pH > 5.5，则整个批次需要返工。

- For other soft treats, 1 mixing sample needs to be taken from every 200 lbs throughout the entire batch during production. A pH > 4.6 results in a rework of the entire batch. 对于其他软食品，在生产过程中，每200磅需要取1个混合样品。 如果pH > 4.6，则整个批次需要返工。

  - Appearance Inspection 外观检验

The defects of different products can be classified into color defects, shape defects, and other defects. Refer to Appendix II (Product appearance inspection SPEC) for details. In short, defects includes不同产品的缺陷可分为颜色缺陷、形状缺陷和其他缺陷。详情请参阅附录II（产品外观检验规范）。简而言之，缺陷包括：

    - Color: light-colored, dark-colored/burnt, uneven color

颜色：浅色、深色/烧焦、颜色不均匀

    - Deform: Release pin hole too deep (over 3mm), bend, flush, under injection, bubble

变形：脱模针孔太深（超过3mm）、弯曲、凹陷、射出不足、气泡

    - Foreign materials: Oily or stained surface, hair or foreign subjects inside the products (can break 2-3 samples to check inside of the products)  异物：产品表面油腻或沾染、产品内部有头发或异物（可破坏2-3个样品以检查产品内部）

    - Crystallization: The moisture and aw of the samples can be too low and the texture of the product is over-hard. 结晶：样品的水分和水活性可能过低，产品质地过硬

  - Water activity and moisture content testing水分活度和水分含量测试

    - Refer to QCD-003 [Moisture Analyzer Operation Instructions] and QCD-005 [Water Activity Meter Operation Instructions] for the specific operation instructions for performing water activity and moisture tests. 执行水活性和水分测试的具体方法，依据《水分测试操作指导》和《水活性操作指导》

    - Check the “Standard for Product Moisture and Water Activity” and judge whether the product is within SPEC.   核对《产品水分及水活性标准》，判断产品特性是否合格

  - pH Testing of Soft Treats软食品的pH测试

    - Prior to the testing of pH, the pH meter needs to be calibrated with pH standards (pH=4.0, and pH=7.0) following below instructions: 在进行pH测试之前，pH计需要使用pH标准物质（pH=4.0和pH=7.0）进行校准，按照以下说明进行：

- Clean the electrodes with deionized water and gently pat dry the electrode with a KIMWIPE.

用去离子水清洁电极，用KIMWIPE轻轻擦干电极。

- Place the electrode in the buffer with a pH value of 7, press the calibration button, and begin reading. 将电极放入pH值为7的缓冲液中，按下校准按钮，并开始读数。

- Set the pH to 7 on the meter and clear previous calibration if needed. 将pH设置为7，并清除先前的校准（如果需要）。

- Rinse the electrode with deionized water and gently pat dry the electrode with a KIMWIPE. 用去离子水冲洗电极，并用KIMWIPE轻轻擦干电极。

- Place the electrode in the buffer with a pH value of 4, press the calibration button, and begin reading. 将电极放入pH值为4的缓冲液中，按下校准按钮，并开始读数。

- Set the pH to 4 on the meter and wait for a stable reading. 将pH设置为4，并等待稳定的读数。

- Rinse the electrode with deionized water and store it in a storage solution of 3.8 M KCl. The pH meter is ready for use at this point. 用去离子水冲洗电极，并将其存放在3.8M KCl的储存液中。此时pH计已准备就绪

    - To measure the pH of the soft treat mixture, the sample needs to be thoroughly blended in a blender, and 20 g will be measured in a beaker with 100 g of DI water added. The mixtures in the beaker needs to be completely mixed and then, the electrode of the pH meter can be placed in the middle of the beaker for a pH reading.     为了测量软食品混合物的pH值，需要将样品彻底混合在搅拌机中，取20克样品并加入100克去离子水。将烧杯中的混合物完全混合，然后将pH计的电极放入烧杯中间进行pH读数。

    - After measurement, the electrode needs to be cleaned with DI water completely, pat dried with KIMWIPE, and then stored in a storage solution of 3.8 M KCl. The blender need to be whipped thoroughly with paper cloth (no water wash until the end of the shift to make sure extra moisture won’t be introduced to the samples). 测量后，电极需要用去离子水彻底清洗，用KIMWIPE擦干，并存放在3.8M KCl的储存液中。搅拌机需要用纸布彻底擦拭（直到班次结束前不使用水清洗，以确保不会向样品引入额外的水分）。

  - Disposal  处置

    - Water activity tests 水活性测试

If the water activity is within the SPEC, stamp QC pass on the sticker and release the product to the next process.    如果水活性符合规格，则在贴纸上盖上QC通过的印章，并将产品放行到下一个流程.

If the product is out of SPEC, put a re-test sticker for all related products and record it in the retest tracking log. Retest all those carts the next day. 如果产品不符合规格，则为所有相关产品贴上重新测试的标签，并在重新测试跟踪日志中记录。次日重新测试所有这些手推车。

    - Appearance Non-conforming 外观不合格:

      - If the semi-product showed unacceptable appearance, an defect rate is needed to be calculated according to the sample plan as described in 4.3.2. Stamp rejection on the sticker and indicates related problems. Notify the supervisor to perform selection, scrap, recycling, etc. according to the treatment method. Submit a non-confirming form and send it to the QC manager. 如果半成品显示不可接受的外观，需要根据4.3.2描述的抽样计划计算缺陷率。在贴纸上盖上拒收的印章并指出相关问题。通知主管进行选择、报废、回收等处理。提交一个不合格表格，并将其发送给QC经理。

      - QC is responsible to re-check the disposal of the non-conformed products and verify the product quality once again after a correction process has been made.   QC负责重新检查不合格产品的处置，并在进行更正后再次验证产品质量。

- Reference Documents/Forms  表单

  - 《Product appearance inspection specification》产品外观检验规范

  - 《Records Management Program》记录管理程序

  - 《Nonconforming Management Program》不合格品管理程序

  - QCD-F17  Corrective Action Report QCD-F17 纠正措施报告

  - QCD-F07 Semi-Product Sample for testing QCD-F07 半成品样本用于测试

- Revision History

01:  07/09/2013

02:  10/15/2013

03: 02/OCT/2018

04: 26/FEB/2019

05: 11/04/2021

06:2/20/2024